Documentation: Shelly Plug PM Gen3 Well-Being Monitor

1. Purpose

This script is designed as a discreet daily well-being signal for an elderly person living alone.
Instead of actively monitoring the person with motion sensors, cameras, or other intrusive devices, it uses a normal daily routine: preparing the morning coffee.
When the coffee maker consumes enough energy, the system assumes that the person is awake and following their usual routine.
A Pushover message is then sent automatically to relatives as a simple “life sign”.
If no coffee preparation is detected by the configured morning check time, an alert is sent so that relatives can follow up.
This approach respects privacy because it does not track movement, presence, or behavior throughout the home.
It only observes a single appliance that is already part of the person’s normal morning habit.
The solution is therefore suitable for supporting independent living while keeping the monitoring as unobtrusive as possible.
Alternative sensors, such as motion detectors, could technically be used, but may be perceived as invasive surveillance.
The coffee maker provides a practical, human-friendly, and privacy-conscious trigger for daily reassurance.

The script can:

- Detect coffee preparation based on consumed energy.

- Send a Pushover notification when coffee has been made.

- Send an emergency Pushover alarm if no coffee has been made by 09:00.

- Change the Shelly Plug LED color depending on current power usage.

- Optionally send an hourly heartbeat status message.

2. Hardware and Services

Required Hardware

- Shelly Plug PM Gen3

- Coffee maker connected to the Shelly Plug

Required External Service

- Pushover account

- Pushover API token

- Pushover user key

The script sends notifications through the Pushover API endpoint:

https://api.pushover.net/1/messages.json

3. Functional Overview

The script works as a daily coffee monitoring system.

At 02:00, it resets its internal state and stores a new energy baseline once the next measurement is available.

From that point onward, it monitors the accumulated energy consumption of the plug. If the energy consumption increases by more than 2.5 Wh, the script assumes that coffee has been prepared.

At 09:00, the script checks whether coffee has been made. If no sufficient consumption has been detected, it sends an emergency Pushover alarm.

The plug LED is also controlled based on the current power draw:

- Below 50 W: low-power LED state

- At or above 50 W: high-power LED state

4. Configuration

Pushover Credentials

let PO_API_TOKEN = "xxxxxxx";
let PO_USER      = "xxxxxxx";

These values must be replaced with the actual Pushover API token and user key.

Notification Messages

let MSG_THRESHOLD      = "☕️ coffee prepared! 👍🏻";
let MSG_NO_CONSUMPTION = "ALARM !!! 🧐 Haven't made any coffee yet!?";

MSG_THRESHOLD is sent when coffee preparation is detected.

MSG_NO_CONSUMPTION is sent if no coffee consumption has been detected by the morning check time.

Pushover Priorities and Sounds

let PRIO_THRESHOLD       = 0;
let SOUND_THRESHOLD      = "magic";

let PRIO_NO_CONSUMPTION  = 2;
let SOUND_NO_CONSUMPTION = "siren";

The coffee-made notification uses normal priority.

The no-consumption alarm uses priority 2, which is Pushover’s emergency priority. For emergency messages, the script also includes retry and expiration parameters.

let PO_RETRY_SEC  = 60;
let PO_EXPIRE_SEC = 3600;

This means the emergency notification is retried every 60 seconds and expires after 3600 seconds.

Energy Thresholds

let ENERGY_THRESHOLD_WH = 2.5;
let MIN_CONSUMPTION_WH  = 2.5;

Both values are currently set to 2.5 Wh.

MIN_CONSUMPTION_WH determines when the script considers that coffee has been made.

ENERGY_THRESHOLD_WH determines when the “coffee prepared” notification is sent.

LED Power Threshold

let POWER_LED_THRESHOLD_W = 50.0;

If the current power draw is below 50 W, the plug is set to the low-power LED state. If the power draw is 50 W or higher, the high-power LED state is used.

Daily Schedule

let RESET_HOUR = 2;
let CHECK_HOUR = 9;

At 02:00, the script resets the daily monitoring state.

At 09:00, the script checks whether coffee has already been made.

Polling Interval

let POLL_INTERVAL_MS = 10 * 1000;

The script polls the Shelly Plug status every 10 seconds.

5. Internal State Variables

let baselineWh        = null;
let hasConsumption    = false;
let notifiedThreshold = false;
let notifiedNoUse     = false;

let lastLedState  = "";
let lastHour      = -1;

baselineWh

Stores the energy meter value after the daily reset. All later consumption is calculated relative to this value.

hasConsumption

Indicates whether enough energy has been consumed to assume coffee was made.

notifiedThreshold

Prevents the “coffee prepared” notification from being sent more than once per day.

notifiedNoUse

Prevents the morning alarm from being sent more than once per day.

lastLedState

Stores the last LED state to avoid unnecessary repeated LED configuration calls.

lastHour

Stores the last processed hour so that hourly actions are triggered only once per hour.

6. Pushover Notifications

Function: sendPushover(message, priority, sound)

This function sends a notification to Pushover.

It builds a JSON payload containing:

- API token

- User key

- Message text

- Priority

- Optional sound

- Emergency retry and expiration values, if priority is 2

Emergency notifications include:

payload.retry  = PO_RETRY_SEC;
payload.expire = PO_EXPIRE_SEC;

The notification is sent using Shelly’s HTTP.POST call.

7. Heartbeat Message

Function: sendHeartbeat(hour)

The heartbeat function prepares a status message containing:

- Current hour

- Last known power value

- Energy consumption since baseline

- Whether coffee has been detected

Example format:

HB 8h W=12.3 dWh=0.8 kaffee=nein

However, in the uploaded script, the actual Pushover send call inside sendHeartbeat() is commented out:

// sendPushover(msg, -2, null);

Therefore, heartbeat messages are currently prepared but not sent.

To enable silent hourly heartbeat notifications, this line must be uncommented.

8. LED Control

Function: setLedState(state)

This function changes the LED configuration of the Shelly Plug.

It supports two states:

Low Power State

Used when power is below 50 W.

rgb = [100, 70, 20];
onBr = 0;
offBr = 0;

High Power State

Used when power is 50 W or higher.

rgb = [100, 40, 5];
onBr = 30;
offBr = 5;

The LED mode is set using:

PLUGPM_UI.SetConfig

The function avoids repeated configuration updates by checking whether the requested LED state is already active.

9. Daily Reset

Function: doDailyReset()

This function resets the monitoring state for a new day.

It clears:

- Energy baseline

- Consumption flag

- Coffee-made notification flag

- No-use alarm flag

After the reset, the next valid energy reading becomes the new baseline.

10. Morning Check

Function: doMorningCheck()

This function checks whether coffee has been made.

If no consumption has been detected and no alarm has already been sent, it sends the configured emergency Pushover notification.

if (!hasConsumption && !notifiedNoUse) {
  notifiedNoUse = true;
  sendPushover(MSG_NO_CONSUMPTION, PRIO_NO_CONSUMPTION, SOUND_NO_CONSUMPTION);
}

11. Hourly Trigger Logic

Function: checkHourTriggers()

This function reads the Shelly system time from:

Shelly.getComponentStatus("sys")

It extracts the current hour from the system time string and triggers actions only when the hour changes.

Execution order:

- Daily reset, if current hour equals RESET_HOUR

- Morning check, if current hour equals CHECK_HOUR

- Heartbeat message

This order ensures that the heartbeat reflects the latest state after reset or morning check.

12. Polling Logic

Function: poll()

The main monitoring function calls:

PM1.GetStatus

It reads the current power and energy values from the Shelly Plug.

Energy Handling

If a valid total energy value is available:

- Store the latest total Wh value.

- Initialize baselineWh if it is still null.

- Calculate the delta:

let delta = totalWh - baselineWh;

- Clamp negative values to zero.

- Mark coffee as made if the delta exceeds MIN_CONSUMPTION_WH.

- Send the coffee-made notification if the delta exceeds ENERGY_THRESHOLD_WH and no notification has been sent yet.

Power Handling

The script reads the current power from:

res.apower

If unavailable, it falls back to:

res.aenergy.power

The power value is then used for LED control.

Time-Based Actions

At the end of each successful polling cycle, the script calls:

checkHourTriggers();

This ensures that reset, morning check, and heartbeat logic are evaluated regularly.

13. Startup Behavior

At the end of the script, a repeating timer is started:

Timer.set(POLL_INTERVAL_MS, true, poll);
poll();

This means:

- poll() is executed immediately after script start.

- poll() is then repeated every 10 seconds.

14. Important Notes

Time Dependency

The script depends on the Shelly system time. The device must have correct time synchronization for the reset and morning check to work correctly.

Energy Threshold

The threshold of 2.5 Wh should be adjusted if the coffee maker uses significantly more or less energy during normal operation.

Heartbeat Disabled by Default

Although the script includes heartbeat logic, the actual notification call is commented out. Heartbeat messages will not be sent unless the sendPushover() call inside sendHeartbeat() is enabled.

Single Notification per Day

The script is designed to send each important notification only once per day:

- One “coffee prepared” notification

- One “no coffee made” alarm

The flags are reset daily at 02:00.

15. Suggested Improvements

There is one formatting issue in the uploaded script around the heartbeat variables:

// Für Heartbeat: letzter bekannter Messwert let lastTotalWh = null; let lastApower  = null;

As written, lastTotalWh and lastApower appear to be part of a comment and may therefore not be declared correctly. They should be written as separate JavaScript statements:

// Last known values for heartbeat
let lastTotalWh = null;
let lastApower  = null;

Without this correction, the script may fail when assigning values to lastTotalWh or lastApower, depending on the Shelly JavaScript runtime behavior.

16. Summary

This script turns a Shelly Plug PM Gen3 into a coffee monitoring system. It uses daily energy consumption to determine whether coffee has been prepared, sends Pushover notifications, provides an emergency reminder if no coffee has been made by 09:00, and uses the plug LED as a visual power indicator.